Y2系列三相异步电动机

Y2 SERIES THREE PHASE induction motor

机座号:63～355        功  率:  0.12～315kW

工作制 :S1      绝缘等级: F  能效等级   IE1（3级）

适用于：一般场所和无特殊要求的机械，如金属切削机床、泵、风机、运输机械、搅拌机、农业机械、食品机械等

Applications: General purpose including cutting machines, pumps, fans, conveyors, machines tools of farm duty and food process

特点：外型美观，噪声低、振动小，绝缘等级为F级，外壳防护等级为IP54或IP55，综合性能与西门子1LA5 6系列相当

Features: High efficiency and energy saving, low noise and little vibration. The insulation class is F; the protective class is IP54 or IP55。The Comprehensive performance is equivalent to siemens 1LA5 6 series motor

YKP系列宽频三相异步电动机

YKP SERIES WIDE FREQUENCY THREE PHASE INDUCTION MOTOR

机座号:63～355        功  率:  0.12～315kW

工作制 :S1                   绝缘等级: F

基准频率：50Hz               调频范围：30～60Hz

适用于：一般场所和无特殊要求的机械，如金属切削机床、泵、风机、运输机械、搅拌机、农业机械、食品机械等。

Applications: General purpose including cutting machines, pumps, fans, conveyors, machines tools of farm duty and food process

特点：外型美观、噪声低、振动小、绝缘等级为F级、外壳防护等级为IP54或IP55、使用频率范围30～60Hz（30～50Hz为变转矩范围，50～60Hz为恒功率范围）

Features: High efficiency and energy saving、low noise and little vibration. The insulation class is F、the protective class is IP54 or IP55、frequency variable range 30-60Hz（30-50Hz for variable torque range, 50-60Hz for constant output range

使用条件：海拔不超过1000m。环境温度随季节变化，但最高不超过+40℃，最低不低于-15℃，无特殊环境要求。

Circumstance For Use: The altitude not exceeding 1000m above sea level. The ambient temperature subject to seasonal variations but not exceeding 40℃ and not less than -15℃，No special environmental requirements.

Y2、YKP性能数据Performance Data                                           380V   50Hz

| 型 号 Type | 功率 kW | 额定 电流 A | 转速 r/min | 效率 Eff.% | 功率 因数 P.F | 额定 转矩 N.m | 堵转转矩 额定转矩 Tst TN | 堵转电流 额定电流 Ist IN | 最大转矩 额定转矩 Tmax TN | 噪声 dB(A) | 重量 kg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 同步转速 3000r/min |  |  |  |  |  |  |  |  |  |  |  |
| Y2、YKP-63M1-2 | 0.18 | 0.53 | 2720 | 65.0 | 0.80 | 0.63 | 2.2 | 5.5 | 2.2 | 61 | 5.5 |
| Y2、YKP -63M2-2 | 0.25 | 0.69 | 2720 | 68.0 | 0.81 | 0.88 | 2.2 | 5.5 | 2.2 | 61 | 6.5 |
| Y2、YKP -71M1-2 | 0.37 | 0.99 | 2740 | 70.0 | 0.81 | 1.29 | 2.2 | 6.1 7.0 | 2.2 | 64 | 9.5 |
| Y2、YKP -71M2-2 | 0.55 | 1.4 | 2740 | 73.0 | 0.82 | 1.92 | 2.2 | 6.1 | 2.3 | 64 | 10.5 |
| Y2、YKP -80M1-2 | 0.75 | 1.8 | 2845 | 75.0 | 0.83 | 2.52 | 2.2 | 6.1 | 2.3 | 67 | 15 |
| Y2、YKP -80M2-2 | 1.1 | 2.6 | 2845 | 77.0 | 0.84 | 3.69 | 2.2 | 7.0 | 2.3 | 67 | 16 |
| Y2、YKP -90S-2 | 1.5 | 3.4 | 2840 | 79.0 | 0.84 | 5.04 | 2.2 | 7.0 | 2.3 | 72 | 20 |
| Y2、YKP -90L-2 | 2.2 | 4.9 | 2840 | 81.0 | 0.85 | 7.40 | 2.2 | 7.0 | 2.3 | 72 | 24 |
| Y2、YKP -100L-2 | 3 | 6.3 | 2860 | 83.0 | 0.87 | 10.0 | 2.2 | 7.5 | 2.3 | 76 | 32 |
| Y2、YKP -112M-2 | 4 | 8.1 | 2880 | 85.0 | 0.88 | 13.3 | 2.2 | 7.5 | 2.3 | 77 | 39 |
| Y2、YKP -132S1-2 | 5.5 | 11.0 | 2900 | 86.0 | 0.88 | 18.1 | 2.2 | 7.5 | 2.3 | 80 | 58 |
| Y2、YKP -132S2-2 | 7.5 | 14.9 | 2900 | 87.0 | 0.88 | 24.7 | 2.2 | 7.5 | 2.3 | 80 | 66 |
| Y2、YKP -160M1-2 | 11 | 21.3 | 2930 | 88.4 | 0.89 | 35.9 | 2.2 | 7.5 | 2.3 | 86 | 104 |
| Y2、YKP -160M2-2 | 15 | 28.7 | 2930 | 89.4 | 0.89 | 48.9 | 2.2 | 7.5 | 2.3 | 86 | 112 |
| Y2、YKP -160L-2 | 18.5 | 34.7 | 2930 | 90.0 | 0.90 | 60.3 | 2.2 | 7.5 | 2.3 | 86 | 132 |
| Y2、YKP -180M-2 | 22 | 41.0 | 2940 | 90.5 | 0.90 | 71.5 | 2.0 | 7.5 | 2.3 | 89 | 162 |
| Y2、YKP -200L1-2 | 30 | 55.4 | 2950 | 91.4 | 0.90 | 97.0 | 2.0 | 7.5 | 2.3 | 92 | 225 |
| Y2、YKP -200L2-2 | 37 | 67.9 | 2950 | 92.0 | 0.90 | 120 | 2.0 | 7.5 | 2.3 | 92 | 245 |
| Y2、YKP -225M-2 | 45 | 82.1 | 2960 | 92.5 | 0.90 | 145 | 2.0 | 7.5 | 2.3 | 92 | 294 |
| Y2、YKP -250M-2 | 55 | 99.8 | 2965 | 93.0 | 0.90 | 177 | 2.0 | 7.5 | 2.3 | 93 | 367 |
| Y2、YKP -280S-2 | 75 | 135 | 2970 | 93.6 | 0.90 | 241 | 2.0 | 7.5 | 2.3 | 94 | 495 |
| Y2、YKP -280M-2 | 90 | 160 | 2970 | 93.9 | 0.91 | 289 | 2.0 | 7.5 | 2.3 | 94 | 541 |
| Y2、YKP -315S-2 | 110 | 195 | 2975 | 94.0 | 0.91 | 353 | 1.8 | 7.1 | 2.2 | 96 | 880 |
| Y2、YKP -315M-2 | 132 | 233 | 2975 | 94.5 | 0.91 | 424 | 1.8 | 7.1 | 2.2 | 96 | 1000 |
| Y2、YKP -315L1-2 | 160 | 279 | 2975 | 94.6 | 0.92 | 514 | 1.8 | 7.1 | 2.2 | 99 | 1080 |
| Y2、YKP -315L2-2 | 200 | 348 | 2975 | 94.8 | 0.92 | 642 | 1.8 | 7.1 | 2.2 | 99 | 1130 |
| Y2、YKP -355M-2 | 250 | 433 | 2980 | 95.3 | 0.92 | 801 | 1.6 | 7.1 | 2.2 | 103 | 1710 |
| Y2、YKP -355L-2 | 315 | 544 | 2980 | 95.6 | 0.92 | 1009 | 1.6 | 7.1 | 2.2 | 103 | 1855 |
| 同步转速 1500r/min |  |  |  |  |  |  |  |  |  |  |  |
| Y2、YKP -63M1-4 | 0.12 | 0.44 | 1310 | 57.0 | 0.72 | 0.87 | 2.1 | 4.4 | 2.2 | 52 | 5 |
| Y2、YKP -63M2-4 | 0.18 | 0.62 | 1310 | 60.0 | 0.73 | 1.31 | 2.1 | 4.4 | 2.2 | 52 | 6.5 |
| Y2、YKP -71M1-4 | 0.25 | 0.79 | 1330 | 65.0 | 0.74 | 1.80 | 2.1 | 5.2 | 2.2 | 55 | 9 |
| Y2、YKP -71M2-4 | 0.37 | 1.12 | 1330 | 67.0 | 0.75 | 2.66 | 2.1 | 5.2 | 2.2 | 55 | 9.5 |
| Y2、YKP -80M1-4 | 0.55 | 1.6 | 1390 | 71.0 | 0.75 | 3.78 | 2.4 | 5.2 | 2.3 | 58 | 15 |
| Y2、YKP -80M2-4 | 0.75 | 2.0 | 1390 | 73.0 | 0.76 | 5.15 | 2.3 | 6.0 | 2.3 | 58 | 16 |
| Y2、YKP -90S-4 | 1.1 | 2.9 | 1390 | 76.2 | 0.77 | 7.56 | 2.3 | 6.0 | 2.3 | 61 | 20 |
| Y2、YKP -90L-4 | 1.5 | 3.7 | 1390 | 78.5 | 0.79 | 10.3 | 2.3 | 6.0 | 2.3 | 61 | 24 |
| Y2、YKP -100L1-4 | 2.2 | 5.1 | 1410 | 81.0 | 0.81 | 14.9 | 2.3 | 7.0 | 2.3 | 64 | 31 |
| Y2、YKP -100L2-4 | 3 | 6.7 | 1410 | 82.6 | 0.82 | 20.3 | 2.3 | 7.0 | 2.3 | 64 | 35 |
| Y2、YKP -112M-4 | 4 | 8.8 | 1440 | 84.2 | 0.82 | 26.5 | 2.3 | 7.0 | 2.3 | 65 | 41 |
| Y2、YKP -132S-4 | 5.5 | 11.7 | 1440 | 85.7 | 0.83 | 36.5 | 2.3 | 7.0 | 2.3 | 71 | 60 |
| Y2、YKP -132M-4 | 7.5 | 15.6 | 1440 | 87.0 | 0.84 | 49.7 | 2.3 | 7.0 | 2.3 | 71 | 74 |
| Y2、YKP -160M-4 | 11 | 22.5 | 1460 | 88.4 | 0.84 | 72.0 | 2.2 | 7.0 | 2.3 | 75 | 108 |
| Y2、YKP -160L-4 | 15 | 30.0 | 1460 | 89.4 | 0.85 | 98.1 | 2.2 | 7.5 | 2.3 | 75 | 128 |
| Y2、YKP -180M-4 | 18.5 | 36.1 | 1470 | 90.5 | 0.86 | 120 | 2.2 | 7.5 | 2.3 | 76 | 158 |
| Y2、YKP -180L-4 | 22 | 42.7 | 1470 | 91.0 | 0.86 | 143 | 2.2 | 7.5 | 2.3 | 76 | 172 |
| Y2、YKP -200L-4 | 30 | 57.6 | 1470 | 92.0 | 0.86 | 195 | 2.2 | 7.2 | 2.3 | 79 | 241 |
| Y2、YKP -225S-4 | 37 | 69.9 | 1475 | 92.5 | 0.87 | 240 | 2.2 | 7.2 | 2.3 | 81 | 285 |
| Y2、YKP -225M-4 | 45 | 84.7 | 1475 | 92.8 | 0.87 | 291 | 2.2 | 7.2 | 2.3 | 81 | 310 |
| Y2、YKP -250M-4 | 55 | 103 | 1480 | 93.0 | 0.87 | 355 | 2.2 | 7.2 | 2.3 | 83 | 375 |
| Y2、YKP -280S-4 | 75 | 140 | 1480 | 93.8 | 0.87 | 484 | 2.2 | 7.2 | 2.3 | 86 | 507 |
| Y2、YKP -280M-4 | 90 | 167 | 1480 | 94.2 | 0.87 | 581 | 2.2 | 7.2 | 2.3 | 86 | 572 |
| Y2、YKP -315S-4 | 110 | 201 | 1480 | 94.5 | 0.88 | 710 | 2.1 | 6.9 | 2.2 | 93 | 930 |
| Y2、YKP -315M-4 | 132 | 240 | 1480 | 94.8 | 0.88 | 852 | 2.1 | 6.9 | 2.2 | 93 | 1050 |
| Y2、YKP -315L1-4 | 160 | 288 | 1480 | 94.9 | 0.89 | 1032 | 2.1 | 6.9 | 2.2 | 97 | 1110 |
| Y2、YKP -315L2-4 | 200 | 359 | 1480 | 95.0 | 0.89 | 1291 | 2.1 | 6.9 | 2.2 | 97 | 1180 |
| Y2、YKP -355M-4 | 250 | 443 | 1490 | 95.3 | 0.90 | 1602 | 2.1 | 6.9 | 2.2 | 101 | 1670 |
| Y2、YKP -355L-4 | 315 | 556 | 1490 | 95.6 | 0.90 | 2019 | 2.1 | 6.9 | 2.2 | 101 | 1830 |
| 同步转速 1000r/min |  |  |  |  |  |  |  |  |  |  |  |
| Y2、YKP -71M1-6 | 0.18 | 0.74 | 850 | 56.0 | 0.66 | 2.02 | 1.9 | 4.0 | 2.0 | 52 | 9 |
| Y2、YKP -71M2-6 | 0.25 | 0.95 | 850 | 59.0 | 0.68 | 2.81 | 1.9 | 4.0 | 2.0 | 52 | 9.5 |
| Y2、YKP -80M1-6 | 0.37 | 1.3 | 885 | 62.0 | 0.70 | 3.99 | 1.9 | 4.7 | 2.0 | 54 | 15 |
| Y2、YKP -80M2-6 | 0.55 | 1.8 | 885 | 65.0 | 0.72 | 5.94 | 1.9 | 4.7 | 2.1 | 54 | 16 |
| Y2、YKP -90S-6 | 0.75 | 2.3 | 910 | 69.0 | 0.72 | 7.87 | 2.0 | 5.5 | 2.1 | 57 | 20 |
| Y2、YKP -90L-6 | 1.1 | 3.2 | 910 | 72.0 | 0.73 | 11.5 | 2.0 | 5.5 | 2.1 | 57 | 24 |
| Y2、YKP -100L-6 | 1.5 | 3.9 | 920 | 76.0 | 0.75 | 15.6 | 2.0 | 5.5 | 2.1 | 61 | 30 |
| Y2、YKP -112M-6 | 2.2 | 5.6 | 935 | 79.0 | 0.76 | 22.5 | 2.0 | 6.5 | 2.1 | 65 | 39 |
| Y2、YKP -132S-6 | 3 | 7.4 | 960 | 81.0 | 0.76 | 29.8 | 2.1 | 6.5 | 2.1 | 69 | 55 |
| Y2、YKP -132M1-6 | 4 | 9.8 | 960 | 82.0 | 0.76 | 39.8 | 2.1 | 6.5 | 2.1 | 69 | 68 |
| Y2、YKP -132M2-6 | 5.5 | 12.9 | 965 | 84.0 | 0.77 | 54.4 | 2.1 | 6.5 | 2.1 | 69 | 73 |
| Y2、YKP -160M-6 | 7.5 | 17.2 | 970 | 86.0 | 0.77 | 73.8 | 2.0 | 6.5 | 2.1 | 73 | 104 |
| Y2、YKP -160L-6 | 11 | 24.5 | 970 | 87.5 | 0.78 | 108 | 2.0 | 6.5 | 2.1 | 73 | 126 |
| Y2、YKP -180L-6 | 15 | 31.6 | 970 | 89.0 | 0.81 | 148 | 2.0 | 7.0 | 2.1 | 73 | 168 |
| Y2、YKP -200L1-6 | 18.5 | 38.6 | 980 | 90.0 | 0.81 | 180 | 2.1 | 7.0 | 2.1 | 76 | 215 |
| Y2、YKP -200L2-6 | 22 | 44.7 | 980 | 90.0 | 0.83 | 214 | 2.1 | 7.0 | 2.1 | 76 | 238 |
| Y2、YKP -225M-6 | 30 | 59.3 | 980 | 91.5 | 0.84 | 292 | 2.0 | 7.0 | 2.1 | 76 | 288 |
| Y2、YKP -250M-6 | 37 | 71.1 | 980 | 92.0 | 0.86 | 361 | 2.1 | 7.0 | 2.1 | 78 | 354 |
| Y2、YKP -280S-6 | 45 | 85.9 | 980 | 92.5 | 0.86 | 439 | 2.1 | 7.0 | 2.0 | 80 | 463 |
| Y2、YKP -280M-6 | 55 | 105 | 980 | 92.8 | 0.86 | 536 | 2.1 | 7.0 | 2.0 | 80 | 508 |
| Y2、YKP -315S-6 | 75 | 142 | 985 | 93.5 | 0.86 | 727 | 2.0 | 7.0 | 2.0 | 85 | 860 |
| Y2、YKP -315M-6 | 90 | 170 | 985 | 93.8 | 0.86 | 873 | 2.0 | 7.0 | 2.0 | 85 | 980 |
| Y2、YKP -315L1-6 | 110 | 207 | 985 | 94.0 | 0.86 | 1066 | 2.0 | 6.7 | 2.0 | 85 | 1060 |
| Y2、YKP -315L2-6 | 132 | 245 | 985 | 94.2 | 0.87 | 1280 | 2.0 | 6.7 | 2.0 | 85 | 1135 |
| Y2、YKP -355M1-6 | 160 | 292 | 990 | 94.5 | 0.88 | 1543 | 1.9 | 6.7 | 2.0 | 92 | 1580 |
| Y2、YKP -355M2-6 | 200 | 365 | 990 | 94.7 | 0.88 | 1929 | 1.9 | 6.7 | 2.0 | 92 | 1710 |
| Y2、YKP 2-355L-6 | 250 | 455 | 990 | 94.9 | 0.88 | 2412 | 1.9 | 6.7 | 2.0 | 92 | 1890 |
| 同步转速 750r/min |  |  |  |  |  |  |  |  |  |  |  |
| Y2、YKP -80M1-8 | 0.18 | 0.88 | 645 | 51.0 | 0.61 | 2.67 | 1.8 | 3.3 | 1.9 | 52 | 15 |
| Y2、YKP -80M2-8 | 0.25 | 1.15 | 645 | 54.0 | 0.61 | 3.70 | 1.8 | 3.3 | 1.9 | 52 | 16 |
| Y2、YKP -90S-8 | 0.37 | 1.49 | 670 | 62.0 | 0.61 | 5.27 | 1.8 | 4.0 | 1.9 | 56 | 20 |
| Y2、YKP -90L-8 | 0.55 | 2.17 | 670 | 63.0 | 0.61 | 7.84 | 1.8 | 4.0 | 2.0 | 56 | 24 |
| Y2、YKP -100L1-8 | 0.75 | 2.4 | 680 | 71.0 | 0.67 | 10.5 | 1.8 | 4.0 | 2.0 | 59 | 28 |
| Y2、YKP -100L2-8 | 1.1 | 3.4 | 680 | 73.0 | 0.69 | 15.4 | 1.8 | 5.0 | 2.0 | 59 | 30 |
| Y2、YKP -112M-8 | 1.5 | 4.4 | 690 | 75.0 | 0.69 | 20.8 | 1.8 | 5.0 | 2.0 | 61 | 38 |
| Y2、YKP -132S-8 | 2.2 | 6 | 705 | 78.0 | 0.71 | 29.8 | 1.8 | 6.0 | 2.0 | 64 | 54 |
| Y2、YKP -132M-8 | 3 | 7.9 | 705 | 79.0 | 0.73 | 40.6 | 1.8 | 6.0 | 2.0 | 64 | 63 |
| Y2、YKP -160M1-8 | 4 | 10.3 | 720 | 81.0 | 0.73 | 53.1 | 1.9 | 6.0 | 2.0 | 68 | 91 |
| Y2、YKP -160M2-8 | 5.5 | 13.6 | 720 | 83.0 | 0.74 | 73.0 | 2.0 | 6.0 | 2.0 | 68 | 103 |
| Y2、YKP -160L-8 | 7.5 | 17.8 | 720 | 85.5 | 0.75 | 99.5 | 2.0 | 6.0 | 2.0 | 68 | 128 |
| Y2、YKP -180L-8 | 11 | 25.1 | 730 | 87.5 | 0.76 | 144 | 2.0 | 6.6 | 2.0 | 70 | 165 |
| Y2、YKP -200L-8 | 15 | 34.1 | 730 | 88.0 | 0.76 | 196 | 2.0 | 6.6 | 2.0 | 73 | 224 |
| Y2、YKP -225S-8 | 18.5 | 41.1 | 730 | 90.0 | 0.76 | 242 | 1.9 | 6.6 | 2.0 | 73 | 255 |
| Y2、YKP -225M-8 | 22 | 47.4 | 730 | 90.5 | 0.78 | 288 | 1.9 | 6.6 | 2.0 | 73 | 292 |
| Y2、YKP -250M-8 | 30 | 63.4 | 735 | 91.0 | 0.79 | 390 | 1.9 | 6.6 | 2.0 | 75 | 368 |
| Y2、YKP -280S-8 | 37 | 77.8 | 735 | 91.5 | 0.79 | 481 | 1.9 | 6.6 | 2.0 | 76 | 475 |
| Y2、YKP -280M-8 | 45 | 94.1 | 735 | 92.0 | 0.79 | 585 | 1.9 | 6.6 | 2.0 | 76 | 527 |
| Y2、YKP -315S-8 | 55 | 111 | 735 | 92.8 | 0.81 | 715 | 1.8 | 6.6 | 2.0 | 82 | 840 |
| Y2、YKP -315M-8 | 75 | 151 | 735 | 93.0 | 0.81 | 974 | 1.8 | 6.6 | 2.0 | 82 | 1020 |
| Y2、YKP -315L1-8 | 90 | 178 | 735 | 93.8 | 0.82 | 1169 | 1.8 | 6.6 | 2.0 | 82 | 1100 |
| Y2、YKP -315L2-8 | 110 | 217 | 735 | 94.0 | 0.82 | 1429 | 1.8 | 6.4 | 2.0 | 82 | 1180 |
| Y2、YKP -355M1-8 | 132 | 261 | 740 | 93.7 | 0.82 | 1704 | 1.8 | 6.4 | 2.0 | 90 | 1610 |
| Y2、YKP -355M2-8 | 160 | 315 | 740 | 94.2 | 0.82 | 2065 | 1.8 | 6.4 | 2.0 | 90 | 1700 |
| Y2、YKP -355L-8 | 200 | 387 | 740 | 94.5 | 0.83 | 2581 | 1.8 | 6.4 | 2.0 | 90 | 1850 |
| 同步转速 600r/min |  |  |  |  |  |  |  |  |  |  |  |
| Y2、YKP -315S-10 | 45 | 99.6 | 590 | 91.5 | 0.75 | 728 | 1.5 | 6.2 | 2.0 | 82 | 830 |
| Y2、YKP -315M-10 | 55 | 121 | 590 | 92.0 | 0.75 | 890 | 1.5 | 6.2 | 2.0 | 82 | 960 |
| Y2、YKP -315L1-10 | 75 | 162 | 590 | 92.5 | 0.76 | 1214 | 1.5 | 6.2 | 2.0 | 82 | 1080 |
| Y2、YKP -315L2-10 | 90 | 191 | 590 | 93.0 | 0.77 | 1457 | 1.5 | 6.2 | 2.0 | 82 | 1115 |
| Y2、YKP -355M1-10 | 110 | 230 | 590 | 93.2 | 0.78 | 1781 | 1.3 | 6.0 | 2.0 | 90 | 1565 |
| Y2、YKP -355M2-10 | 132 | 275 | 590 | 93.5 | 0.78 | 2137 | 1.3 | 6.0 | 2.0 | 90 | 1685 |
| Y2、YKP -355L-10 | 160 | 333 | 590 | 93.5 | 0.78 | 2590 | 1.3 | 6.0 | 2.0 | 90 | 1830 |

外形及安装尺寸  Dimensions                                                 mm

B14 (机座无底脚、端盖上有小凸缘)

B34 (机座带底脚、端盖上有小凸缘)

| 机座号 Frame | 极数 poles | 安装尺寸 Mounting Dimension |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 外形尺寸 Overall Dimension |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D | E | F | G | H | K | M | N | P | R | S | T | 凸缘孔数 | AA | AB | AC | AD | BB | HA | HD | L |
| 63M | 2.4 | 110 | 80 | 40 | 11 | 23 | 4 | 8.5 | 63 | 7 | 75 | 60 | 90 | 0 | M5 | 2.5 | 4 | 30 | 135 | 130 | 70 | 110 | 8 | 180 | 230 |
| 71M | 2.4.6 | 112 | 90 | 45 | 14 | 30 | 5 | 11 | 71 | 7 | 85 | 70 | 105 | 0 | M6 | 2.5 | 4 | 32 | 150 | 145 | 80 | 120 | 8 | 195 | 255 |
| 80M | 2.4.6.8 | 125 | 100 | 50 | 19 | 40 | 6 | 15.5 | 80 | 10 | 100 | 80 | 120 | 0 | M6 | 3 | 4 | 32 | 165 | 175 | 145 | 130 | 10 | 214 | 295 |
| 90S | 2.4.6.8 | 140 | 100 | 56 | 24 | 50 | 8 | 20 | 90 | 10 | 115 | 95 | 140 | 0 | M8 | 3 | 4 | 34 | 180 | 195 | 155 | 140 | 12 | 250 | 320 |
| 90L | 2.4.6.8 | 140 | 125 | 56 | 24 | 50 | 8 | 20 | 90 | 10 | 115 | 95 | 140 | 0 | M8 | 3 | 4 | 34 | 180 | 195 | 155 | 165 | 12 | 250 | 345 |
| 100L | 2.4.6.8 | 160 | 140 | 63 | 28 | 60 | 8 | 24 | 100 | 12 | 130 | 110 | 160 | 0 | M8 | 3.5 | 4 | 39 | 205 | 215 | 180 | 186 | 14 | 270 | 385 |
| 112M | 2.4.6.8 | 190 | 140 | 70 | 28 | 60 | 8 | 24 | 112 | 12 | 130 | 110 | 160 | 0 | M8 | 3.5 | 4 | 45 | 230 | 240 | 190 | 180 | 13 | 300 | 400 |

B3（机座带底脚、端盖无凸缘）

外形及安装尺寸  Dimensions                                                 mm

B5（机座不带底脚、端盖有凸缘）

| 机座号 Frame | 极数 poles | 安装尺寸 Mounting Dimension |  |  |  |  |  |  |  |  |  |  | 外形尺寸 Overall Dimension |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | D | E | F | G | M | N | P | R | S | T | 凸缘孔数 | AC | AD | HF | L |
| 63M | 2.4 | 11 | 23 | 4 | 8.5 | 115 | 95 | 140 | 0 | 10 | 3 | 4 | 130 | 70 | 130 | 230 |
| 71M | 2.4.6 | 14 | 30 | 5 | 11 | 130 | 110 | 160 | 0 | 10 | 3.5 | 4 | 145 | 80 | 145 | 255 |
| 80M | 2.4.6.8 | 19 | 40 | 6 | 15.5 | 165 | 130 | 200 | 0 | 12 | 3.5 | 4 | 175 | 145 | 185 | 295 |
| 90S | 2.4.6.8 | 24 | 50 | 8 | 20 | 165 | 130 | 200 | 0 | 12 | 3.5 | 4 | 195 | 155 | 195 | 320 |
| 90L | 2.4.6.8 | 24 | 50 | 8 | 20 | 165 | 130 | 200 | 0 | 12 | 3.5 | 4 | 195 | 155 | 195 | 345 |
| 100L | 2.4.6.8 | 28 | 60 | 8 | 24 | 215 | 180 | 250 | 0 | 14.5 | 4 | 4 | 215 | 180 | 245 | 385 |
| 112M | 2.4.6.8 | 28 | 60 | 8 | 24 | 215 | 180 | 250 | 0 | 14.5 | 4 | 4 | 240 | 190 | 265 | 400 |
| 132S | 2.4.6.8 | 38 | 80 | 10 | 33 | 265 | 230 | 300 | 0 | 14.5 | 4 | 4 | 275 | 210 | 315 | 470 |
| 132M | 2.4.6.8 | 38 | 80 | 10 | 33 | 265 | 230 | 300 | 0 | 14.5 | 4 | 4 | 275 | 210 | 315 | 510 |
| 160M | 2.4.6.8 | 42 | 110 | 12 | 37 | 300 | 250 | 350 | 0 | 18.5 | 5 | 4 | 330 | 255 | 385 | 615 |
| 160L | 2.4.6.8 | 42 | 110 | 12 | 37 | 300 | 250 | 350 | 0 | 18.5 | 5 | 4 | 330 | 255 | 385 | 670 |
| 180M | 2.4.6.8 | 48 | 110 | 14 | 42.5 | 300 | 250 | 350 | 0 | 18.5 | 5 | 4 | 380 | 280 | 430 | 700 |
| 180L | 2.4.6.8 | 48 | 110 | 14 | 42.5 | 300 | 250 | 350 | 0 | 18.5 | 5 | 4 | 380 | 280 | 430 | 740 |
| 200L | 2.4.6.8 | 55 | 110 | 16 | 49 | 350 | 300 | 400 | 0 | 18.5 | 5 | 4 | 420 | 305 | 480 | 770 |
| 225S | 4.8 | 60 | 140 | 18 | 53 | 400 | 350 | 450 | 0 | 18.5 | 5 | 8 | 470 | 335 | 535 | 815 |
| 225M | 2 | 55 | 110 | 16 | 49 | 400 | 350 | 450 | 0 | 18.5 | 5 | 8 | 470 | 335 | 535 | 820 |
| 225M | 4.6.8 | 60 | 140 | 18 | 53 | 400 | 350 | 450 | 0 | 18.5 | 5 | 8 | 470 | 335 | 535 | 845 |
| 250M | 2 | 60 | 140 | 18 | 53 | 500 | 450 | 550 | 0 | 18.5 | 5 | 8 | 510 | 370 | 595 | 910 |
| 250M | 4.6.8 | 65 | 140 | 18 | 58 | 500 | 450 | 550 | 0 | 18.5 | 5 | 8 | 510 | 370 | 595 | 910 |
| 280S | 2 | 65 | 140 | 18 | 58 | 500 | 450 | 550 | 0 | 18.5 | 5 | 8 | 580 | 410 | 650 | 985 |
| 280S | 4.6.8 | 75 | 140 | 20 | 67.5 | 500 | 450 | 550 | 0 | 18.5 | 5 | 8 | 580 | 410 | 650 | 985 |
| 280M | 2 | 65 | 140 | 18 | 58 | 500 | 450 | 550 | 0 | 18.5 | 5 | 8 | 580 | 410 | 650 | 1035 |
| 280M | 4.6.8 | 75 | 140 | 20 | 67.5 | 500 | 450 | 550 | 0 | 18.5 | 5 | 8 | 580 | 410 | 650 | 1035 |

B35（机座带底脚、端盖有凸缘）

外形及安装尺寸  Dimensions                                                   mm

| 机座号 Frame | 极数 poles | 安装尺寸 Mounting Dimension |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 外形尺寸 Overall Dimension |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D | E | F | G | H | K | M | N | P | R | S | T | AA | AB | AC | AD | BB | HA | HD | L |
| 63M | 2.4 | 100 | 80 | 40 | 11 | 23 | 4 | 8.5 | 63 | 7 | 115 | 95 | 140 | 0 | 4-Φ10 | 3 | 30 | 135 | 130 | 70 | 110 | 8 | 180 | 230 |
| 71M | 2.4.6 | 112 | 90 | 45 | 14 | 30 | 5 | 11 | 71 | 7 | 130 | 110 | 160 | 0 | 4-Φ10 | 3.5 | 32 | 150 | 145 | 80 | 120 | 8 | 195 | 255 |
| 80M | 2.4.6.8 | 125 | 100 | 50 | 19 | 40 | 6 | 15.5 | 80 | 10 | 165 | 130 | 200 | 0 | 4-Φ12 | 3.5 | 32 | 165 | 175 | 145 | 130 | 10 | 220 | 295 |
| 90S | 2.4.6.8 | 140 | 100 | 56 | 24 | 50 | 8 | 20 | 90 | 10 | 165 | 130 | 200 | 0 | 4-Φ12 | 3.5 | 34 | 180 | 195 | 155 | 140 | 12 | 250 | 320 |
| 90L | 2.4.6.8 | 140 | 125 | 56 | 24 | 50 | 8 | 20 | 90 | 10 | 165 | 130 | 200 | 0 | 4-Φ12 | 3.5 | 34 | 180 | 195 | 155 | 165 | 12 | 250 | 345 |
| 100L | 2.4.6.8 | 160 | 140 | 63 | 28 | 60 | 8 | 24 | 100 | 12 | 215 | 180 | 250 | 0 | 4-Φ14.5 | 4 | 39 | 205 | 215 | 180 | 186 | 14 | 270 | 385 |
| 112M | 2.4.6.8 | 190 | 140 | 70 | 28 | 60 | 8 | 24 | 112 | 12 | 215 | 180 | 250 | 0 | 4-Φ14.5 | 4 | 45 | 230 | 240 | 190 | 180 | 13 | 300 | 400 |
| 132S | 2.4.6.8 | 216 | 140 | 89 | 38 | 80 | 10 | 33 | 132 | 12 | 265 | 230 | 300 | 0 | 4-Φ14.5 | 4 | 55 | 270 | 275 | 210 | 186 | 20 | 345 | 470 |
| 132M | 2.4.6.8 | 216 | 178 | 89 | 38 | 80 | 10 | 33 | 132 | 12 | 265 | 230 | 300 | 0 | 4-Φ14.5 | 4 | 55 | 270 | 275 | 210 | 224 | 20 | 345 | 510 |
| 160M | 2.4.6.8 | 254 | 210 | 108 | 42 | 110 | 12 | 37 | 160 | 14.5 | 300 | 250 | 350 | 0 | 4-Φ18.5 | 5 | 65 | 320 | 330 | 255 | 260 | 20 | 420 | 615 |
| 160L | 2.4.6.8 | 254 | 254 | 108 | 42 | 110 | 12 | 37 | 160 | 14.5 | 300 | 250 | 350 | 0 | 4-Φ18.5 | 5 | 65 | 320 | 330 | 255 | 304 | 20 | 420 | 670 |
| 180M | 2.4.6.8 | 279 | 241 | 121 | 48 | 110 | 14 | 42.5 | 180 | 14.5 | 300 | 250 | 350 | 0 | 4-Φ18.5 | 5 | 70 | 355 | 380 | 280 | 311 | 22 | 455 | 700 |
| 180L | 2.4.6.8 | 279 | 279 | 121 | 48 | 110 | 14 | 42.5 | 180 | 14.5 | 300 | 250 | 350 | 0 | 4-Φ18.5 | 5 | 70 | 355 | 380 | 280 | 349 | 22 | 455 | 740 |
| 200L | 2.4.6.8 | 318 | 305 | 133 | 55 | 110 | 16 | 49 | 200 | 18.5 | 350 | 300 | 400 | 0 | 4-Φ18.5 | 5 | 74 | 395 | 420 | 305 | 379 | 26 | 505 | 770 |
| 225S | 4.8 | 356 | 286 | 149 | 60 | 140 | 18 | 53 | 225 | 18.5 | 400 | 350 | 450 | 0 | 8-Φ18.5 | 5 | 78 | 435 | 470 | 335 | 368 | 28 | 560 | 815 |
| 225M | 2 | 356 | 311 | 149 | 55 | 110 | 16 | 49 | 225 | 18.5 | 400 | 350 | 450 | 0 | 8-Φ18.5 | 5 | 78 | 435 | 470 | 335 | 393 | 28 | 560 | 820 |
| 225M | 4.6.8 | 356 | 311 | 149 | 60 | 140 | 18 | 53 | 225 | 18.5 | 400 | 350 | 450 | 0 | 8-Φ18.5 | 5 | 78 | 435 | 470 | 335 | 393 | 28 | 560 | 845 |
| 250M | 2 | 406 | 349 | 168 | 60 | 140 | 18 | 53 | 250 | 24 | 500 | 450 | 550 | 0 | 8-Φ18.5 | 5 | 80 | 490 | 510 | 370 | 445 | 30 | 615 | 910 |
| 250M | 4.6.8 | 406 | 349 | 168 | 65 | 140 | 18 | 58 | 250 | 24 | 500 | 450 | 550 | 0 | 8-Φ18.5 | 5 | 80 | 490 | 510 | 370 | 445 | 30 | 615 | 910 |
| 280S | 2 | 457 | 368 | 190 | 65 | 140 | 18 | 58 | 280 | 24 | 500 | 450 | 550 | 0 | 8-Φ18.5 | 5 | 90 | 550 | 580 | 410 | 485 | 35 | 680 | 985 |
| 280S | 4.6.8 | 457 | 368 | 190 | 75 | 140 | 20 | 67.5 | 280 | 24 | 500 | 450 | 550 | 0 | 8-Φ18.5 | 5 | 90 | 550 | 580 | 410 | 485 | 35 | 680 | 985 |
| 280M | 2 | 457 | 419 | 190 | 65 | 140 | 18 | 58 | 280 | 24 | 500 | 450 | 550 | 0 | 8-Φ18.5 | 5 | 90 | 550 | 580 | 410 | 536 | 35 | 680 | 1035 |
| 280M | 4.6.8 | 457 | 419 | 190 | 75 | 140 | 20 | 67.5 | 280 | 24 | 500 | 450 | 550 | 0 | 8-Φ18.5 | 5 | 90 | 550 | 580 | 410 | 536 | 35 | 680 | 1035 |
| 315S | 2 | 508 | 406 | 216 | 65 | 140 | 18 | 58 | 315 | 28 | 600 | 550 | 660 | 0 | 8-Φ24 | 6 | 120 | 635 | 645 | 530 | 570 | 45 | 845 | 1240 |
| 315S | 4.6.8.10 | 508 | 406 | 216 | 80 | 170 | 22 | 71 | 315 | 28 | 600 | 550 | 660 | 0 | 8-Φ24 | 6 | 120 | 635 | 645 | 530 | 570 | 45 | 845 | 1270 |
| 315M | 2 | 508 | 457 | 216 | 65 | 140 | 18 | 58 | 315 | 28 | 600 | 550 | 660 | 0 | 8-Φ24 | 6 | 120 | 635 | 645 | 530 | 610 | 45 | 845 | 1350 |
| 315M | 4.6.8.10 | 508 | 457 | 216 | 80 | 170 | 22 | 71 | 315 | 28 | 600 | 550 | 660 | 0 | 8-Φ24 | 6 | 120 | 635 | 645 | 530 | 610 | 45 | 845 | 1380 |
| 315L | 2 | 508 | 508 | 216 | 65 | 140 | 18 | 58 | 315 | 28 | 600 | 550 | 660 | 0 | 8-Φ24 | 6 | 120 | 635 | 645 | 530 | 680 | 45 | 845 | 1350 |
| 315L | 4.6.8.10 | 508 | 508 | 216 | 80 | 170 | 22 | 71 | 315 | 28 | 600 | 550 | 660 | 0 | 8-Φ24 | 6 | 120 | 635 | 645 | 530 | 680 | 45 | 845 | 1380 |
| 355M | 2 | 610 | 560 | 254 | 75 | 140 | 20 | 67.5 | 355 | 28 | 740 | 680 | 800 | 0 | 8-Φ24 | 6 | 130 | 730 | 710 | 655 | 690 | 52 | 1010 | 1500 |
| 355M | 4.6.8.10 | 610 | 560 | 254 | 95 | 170 | 25 | 86 | 355 | 28 | 740 | 680 | 800 | 0 | 8-Φ24 | 6 | 130 | 730 | 710 | 655 | 690 | 52 | 1010 | 1530 |
| 355L | 2 | 610 | 630 | 254 | 75 | 140 | 20 | 67.5 | 355 | 28 | 740 | 680 | 800 | 0 | 8-Φ24 | 6 | 130 | 730 | 710 | 655 | 750 | 52 | 1010 | 1500 |
| 355L | 4.6.8.10 | 610 | 630 | 254 | 95 | 170 | 25 | 86 | 355 | 28 | 740 | 680 | 800 | 0 | 8-Φ24 | 6 | 130 | 730 | 710 | 655 | 750 | 52 | 1010 | 1530 |

V1（立式安装、机座不带底脚、端盖有凸缘、轴伸向下的电动机）

外形及安装尺寸  Dimensions                                                     mm

| 机座号 Frame | 极数 poles | 安 装 尺 寸 Mounting Dimension |  |  |  |  |  |  |  |  |  | 外形尺寸 Overall Dimension |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | D | E | F | G | M | N | P | R | S | T | AC | AD | HF | L |
| 180M | 2.4.6.8 | 48 | 110 | 14 | 42.5 | 300 | 250 | 350 | 0 | 4-Φ18.5 | 5 | 380 | 280 | 500 | 760 |
| 180L | 2.4.6.8 | 48 | 110 | 14 | 42.5 | 300 | 250 | 350 | 0 | 4-Φ18.5 | 5 | 380 | 280 | 500 | 800 |
| 200L | 2.4.6.8 | 55 | 110 | 16 | 49 | 350 | 300 | 400 | 0 | 4-Φ18.5 | 5 | 420 | 305 | 550 | 840 |
| 225S | 4.8 | 60 | 140 | 18 | 53 | 400 | 350 | 450 | 0 | 8-Φ18.5 | 5 | 470 | 335 | 610 | 905 |
| 225M | 2 | 55 | 110 | 16 | 49 | 400 | 350 | 450 | 0 | 8-Φ18.5 | 5 | 470 | 335 | 610 | 910 |
| 225M | 4.6.8 | 60 | 140 | 18 | 53 | 400 | 350 | 450 | 0 | 8-Φ18.5 | 5 | 470 | 335 | 610 | 935 |
| 250M | 2 | 60 | 140 | 18 | 53 | 500 | 450 | 550 | 0 | 8-Φ18.5 | 5 | 510 | 370 | 650 | 1015 |
| 250M | 4.6.8 | 65 | 140 | 18 | 58 | 500 | 450 | 550 | 0 | 8-Φ18.5 | 5 | 510 | 370 | 650 | 1015 |
| 280S | 2 | 65 | 140 | 18 | 58 | 500 | 450 | 550 | 0 | 8-Φ18.5 | 5 | 580 | 410 | 720 | 1110 |
| 280S | 4.6.8 | 75 | 140 | 20 | 67.5 | 500 | 450 | 550 | 0 | 8-Φ18.5 | 5 | 580 | 410 | 720 | 1110 |
| 280M | 2 | 65 | 140 | 18 | 58 | 500 | 450 | 550 | 0 | 8-Φ18.5 | 5 | 580 | 410 | 720 | 1150 |
| 280M | 4.6.8 | 75 | 140 | 20 | 67.5 | 500 | 450 | 550 | 0 | 8-Φ18.5 | 5 | 580 | 410 | 720 | 1150 |
| 315S | 2 | 65 | 140 | 18 | 58 | 600 | 550 | 660 | 0 | 8-Φ24 | 6 | 645 | 530 | 900 | 1360 |
| 315S | 4.6.8.10 | 80 | 170 | 22 | 71 | 600 | 550 | 660 | 0 | 8-Φ24 | 6 | 645 | 530 | 900 | 1390 |
| 315M | 2 | 65 | 140 | 18 | 58 | 600 | 550 | 660 | 0 | 8-Φ24 | 6 | 645 | 530 | 900 | 1470 |
| 315M | 4.6.8.10 | 80 | 170 | 22 | 71 | 600 | 550 | 660 | 0 | 8-Φ24 | 6 | 645 | 530 | 900 | 1510 |
| 315L | 2 | 65 | 140 | 18 | 58 | 600 | 550 | 660 | 0 | 8-Φ24 | 6 | 645 | 530 | 900 | 1470 |
| 315L | 4.6.8.10 | 80 | 170 | 22 | 71 | 600 | 550 | 660 | 0 | 8-Φ24 | 6 | 645 | 530 | 900 | 1510 |
| 355M | 2 | 75 | 140 | 20 | 67.5 | 740 | 680 | 800 | 0 | 8-Φ24 | 6 | 710 | 655 | 1010 | 1640 |
| 355M | 4.6.8.10 | 95 | 170 | 25 | 86 | 740 | 680 | 800 | 0 | 8-Φ24 | 6 | 710 | 655 | 1010 | 1670 |
| 355L | 2 | 75 | 140 | 20 | 67.5 | 740 | 680 | 800 | 0 | 8-Φ24 | 6 | 710 | 655 | 1010 | 1640 |
| 355L | 4.6.8.10 | 95 | 170 | 25 | 86 | 740 | 680 | 800 | 0 | 8-Φ24 | 6 | 710 | 655 | 1010 | 1670 |

- 表中外形尺寸为电机最大外形尺寸，实际情况以实物为准。 Data subject to change without notice

常用的安装结构型式，以及适用的机座号见下表

Conventional mounting type and suitable frame size are given in following table（with “√”)

| 机座号 Frame | 基本安装结构 basic type |  |  | 派生的安装形式 derived type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B3 | B5 | B35 | V1 | V3 | V5 | V6 | B6 | B7 | B8 | V15 | V17 | V35 | V37 | B14 | B34 | V18 | V19 |
| 63～112 | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ |
| 132～160 | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | - | - | - | - |
| 180～280 | √ | √ | √ | √ | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 315～355 | √ | - | √ | √ | - | - | - | - | - | - | - | - | - | - | - | - | - | - |

“√”表示可以制造生产的结构型式。