此类调压器是一种可根据客户的实际需要压力而随意调整的调压器，有电动式和手轮式两种式样，I-72-1 入口压力：0.13~1.56MPA,出口压力设定范围：0.01~0.07MPA,进口连接：POL阳，出口连接:RC1/2,流量范围：3~5立方/小时（空气） I-72-2 入口压力：0.17~1.56MPA,出口压力设定范围：0.01~0.16MPA,进口连接：POL阳，出口连接:RC1/2,流量范围：3~5立方/小时（空气）    I-72-3 入口压力：0.40~1.56MPA,出口压力设定范围：0.01~0.35MPA,进口连接：POL阳，出口连接:RC1/2,流量范围：3~5立方/小时（空气）

| 型号 | 进口压力（Mpa） | 出口压力 | 进口连接 | 出口连接 | 最大流量（AIR) | 尺寸（mm） |
| --- | --- | --- | --- | --- | --- | --- |
| I-72-1 | 0.13~1.56 | 0.01~0.07Mpa | POL阳 | Rc1/2 | 5Nm3/h |  |
| I-72-2 | 0.17~1.56 | 0.01~0.16Mpa |  |  | 5Nm3/h | 206 |
| I-72-3 | 0.40~1.56 | 0.01~0.35Mpa |  |  | 5Nm3/h | 280 |
| CM-100-H1 | 0.1~1.56 | 20~300Kpa | JIS20K 15A法兰 | JIS10K 15A法兰 | 30Nm3/h | 365 |
| GM-16A-H1 |  |  | JIS20K 20A法兰 | JIS10K 20A法兰 | 70Nm3/h | 365 |
| CM-100-H2 | 0.1~0.4 | 2~40Kpa | JIS20K 20A法兰 | JIS10K 20A法兰 | 30Nm3/h | 420 |
| GM-16A-H2 |  |  | JIS20K25A法兰 | JIS10K 25A法兰 | 70Nm3/h | 490 |

上海寅驰自动化控制设备有限公司为日本伊藤华东地区总经销

集团公司伊藤工机株式会社自1953年4月创立以来，已有60年历史，本着“品质第一”的原则，稳步发展壮大。作为日本顶尖的燃气供给相关设备制造厂商的同时，又是一名海外市场的积极开拓者。在构筑“安全、安心、舒适的燃气生活”过程中，我们将不懈努力，永无止尽。

集团公司伊藤工机株式会社拥有强大的技术阵营（拥有40余名技术研发人员），在60年的发展历程中，集团公司在日本及世界得到了广泛的认可，取得为数众多的产品研究、开发技术奖项，于1972年获得“通商产业大臣认定事业所”的称号，1998年取得ISO9001认定证书。致力于环境友好，于2005年取得了ISO14001认证。

正是由于伊藤工机品牌产品的优越性能——高安全性、高信赖性，使得集团公司产品在世界范围内，尤其是在东南亚国家，树立了良好的声誉。

产品型号：

单段式调压器（燃气高压到低压减压阀）

HS-5A(接钢瓶) C-10-A-1 C-20-1（法兰） C-20-1(螺纹) GL-50-1 GL-70-1 G-32A-1 G-36C-1

二段式一次调压器（燃气高压到中压减压阀）

CM-100 GM-16A GMF-200 GMF-300 GMF-400 GMF-500 GMF-1000

二段式二次用调压器（燃气中压到低压减压阀）

C-10A-2 C-20-2(法兰) C-20-2(螺纹) GL-50-2 GL-70-2 G-32A-2 G-36C-2 GL-300-2 GL-400-2 GL-500-2

可变压力式调压器（出口调节范围大）

I-72-1 I-72-2 I-72-3 CM-100-H1 CM-100-H2 GM-16A-H1 GM-16A-H2 GM-16A-M GMF-100-M G-32A-M G-36C-M GL-500-M

SGX整压阀

SGX-10A SGX-15N SGX-20N SGX-25N SGX-40 SGX-50

耐高温SGX稳压阀（COG高炉煤气等气体用）

SGX-10AC SGX-15NC SGX-20NC SGX-25NC SGX-40-C SGX-50-C

出口压力可调节SGX整压阀(0.5-4kpa)

SGX-15H SGX-20H SGX-25H(NH)

燃气SGX零压阀（均压阀）

SGX-10ZA SGX-15NZ SGX-20NZ SGX-25NZ SGX-40NZ SGX-40Z SGX-50Z SGX-24AZ SGX-32BZ SGX-36AZ

耐高温焦炉煤气SGX零压阀（均压阀）

SGX-10ZA-C SGX-15NZ-C SGX-20NZ-C SGX-25NZ-C SGX-40NZ-C SGX-40Z-C SGX-50Z-C SGX-24AZ-C SGX-32BZ-C SGX-36AZ-C

煤气双阀体高性能减压阀

NGX-20 NGX-25 NGX-32 NGX-50 NGX-50-2 NGX-50-3R NGX-80 NGX-100 NGX-100-3 NGX-100-3R

不锈钢调压器（氨气减压阀）

AC-15 AM-150 GM-16A-H1 GM-16A-H2

感震式自动燃气切断装置

C2-250-B C2-400-B

放散阀

CM-100-R GM-16A-R SGX-15R SGX-20R SGX-25R SGX-40R SGX-50R SGX-24-R SGX-32B-R SGX-36A-R

液化气天然气两用调压器H5-EX(S) H5-EX(S)-NR低低压天然气调压器 SGF-15 户内调压器IGL-20H IGL-50H

PA空气吸入式可移动燃气发生装置

PA-13A4N PA-12A30N PA-60

自动切换式一体型调压器

AXS-8B AXS-10A AX-20A AX-30A AX-50A  自动切换分离式调压器AX-30A-0 AX-50-0 AX-160-0 AXF-100-0 AXF-200-0 AXF-300-0 AXF-400-0 AXF-500-0可调型自动切换式调压器规格AX-10AH1 AX-10AH2 AX-10M AX-30AM带发信号功能自动切换式调压器TAXS-80 TAXS-10A TAX-20A TAX-30A TAX-50监视压力式检知泄漏装置HLB-3 HLB-5

液相自动切换阀（LAX）LAX-200B LAX-20C LAX-20B-R(可发信号)LAX-20C-R(可发信号)

电热温水式气化器（s型）规格 S-50 S-100 S-150 S-200 S-300 S-500 S-600温水循环式壁挂型气化器（SMV系列）规格SMV-30C SMV-50C SMV-100C空温式气化器（SR系列）规格SR-30SK SR-50SK SR-100ASK SR-200SK

液化气用高压橡胶软管LIHS-6-10H HIS-6-10HK