DD5大容量离心机

Model: DD5 large capacity centrifuge

Application:

Widely used in the field of biochemistry、biological products、pharmaceutical factory and laboratory.

Features:

The digital display which indicates the speed、time and temperature.

Micro computer control, there are 10 kinds of program and 10 kinds of acceleration and deceleration for your choice.

There are 8 kinds of rotors for your choice.

Electric lid lock、over speed protection 、over temperature protection and imbalance protection.

The centrifuge body is made of high quality steel, safe and reliable.

仪器特点：

。采用微机处理器精准控制，数字显示转速、温度、时间等参数，按键编程，10种升降速率供选择。

。门盖独立电机伺服，减轻操作人员劳动强度，超速、超温自动保护，不平衡保护，机身采用优质钢材结构；安全可靠，使用倍感方便。

。采用本公司特有的弹簧锥套连接转头与主轴，装卸转头快捷简单，无方向性，安全可靠，使用倍感方便。

。配有多种转头，并可根据试验要求设计各种适配器，一机多用。

。三级减振，离心效果达到最佳。

DD5技术参数：

| 最高转速max speed： | 5000r/min | 最大离心力max RCF： | 3860×g |
| --- | --- | --- | --- |
| 最大容量max volume： | 4×800ml | 电 源power supply： | AC 220V 50HZ 10A |
| 定时范围timer： | 1～99min | 噪 音noise： | ≤58dBA |
| 外型尺寸DD5dimension： | 655×500×790 | 净 重DD5net weight： | 108KG |
| 控速精度speed accuracy： | ±50r/min | 净重TD5Bnet weight： | 67KG |

注册证号：湘长食药监械（准）字2007第1410043 号

DD5大容量离心机适配转头

Rotors for centrifuge DD5

| 编号 | 转子类型 | 最高转速r/min | 容量ml | 最大离心力×g |
| --- | --- | --- | --- | --- |
| No31495 | 水平转子(圆杯) | 4000 | 2×800ml | 3380 |
| No31496 | 水平转子(圆杯) | 4000 | 4×800ml | 3380 |
|  | 适配40301—40308适配器 |  |  |  |
|  | 水平转子(椭圆杯) | 4000 | 4×500ml | 3380 |
| No31392 | 水平转子(圆杯) | 4000 | 4×300ml(250ml试杯) | 2920 |
|  | 适配40401—40408适配器 |  |  |  |
| No31389 | 水平转子 | 4000 | 4×22×5ml(真空管) | 2840 |
|  |  |  | 4×31×5ml(放免管) |  |
| No31390 | 水平转子 | 4000 | 4×22×7ml(真空管) | 3140 |
|  |  |  | 4×31×7ml(放免管) |  |
| No31491 | 水平转子(酶标板) | 4000 | 2×4×96孔 | 2490 |
| No31494 | 水平转子(酶标板) | 4000 | 4×4×96孔 | 2840 |
| No31376 | 水平转子(酶标板) | 4000 | 2×3×48孔 | 2300 |
| No31479 | 水平转子(方杯) | 4000 | 4×500ml | 3310 |
|  | 适配40101—40108适配器 |  |  |  |
| No31493 | 水平转子 | 4000 | 乳脂瓶 | 3830 |
| No30627 | 角转子 | 5000 | 30×15ml | 3830 |
| No30638 | 角转子 | 5000 | 6×15ml | 2540 |
| No30607 | 角转子 | 5000 | 12×15ml | 3080 |
| No30639 | 角转子 | 5000 | 24×15ml | 3500 |
| No30640 | 角转子 | 5000 | 4×50ml | 2520 |
| No30611 | 角转子 | 5000 | 6×50ml | 2850 |
| No30641 | 角转子 | 5000 | 12×50ml | 3860 |
| No30642 | 角转子 | 4000 | 24×50ml | 2970 |
| No30613 | 角转子 | 5000 | 4×100ml | 2630 |
| No30614 | 角转子 | 5000 | 6×100ml | 3130 |
| No30643 | 角转子 | 4000 | 12×100ml | 2970 |